SIMATS ENGINEERING (SAVEETHA SCHOOL OF ENGINEERING)

Department of Computer Science and Engineering

ASSESSMENT TOOL 3 – INDIVIDUAL TALK

| Course Code: | ITA0324 | Course Title: | Mobile computing |
| --- | --- | --- | --- |
| CO Assessed: | CO1 | Assessment: | Assessment Tool 3 – INDIVIDUAL TALK |
| Weightage: | 30 % of CIA | Total Marks: | 20 marks |
| CO1 Statement: | Analyse the mobile network components, mobile base station, mobile infrastructures and protocols in cellular systems. |  |  |
| Student Name: | Y. Neeraj | Reg. No.: | 192425043 |
| Section / Batch: | CSE-AI/ML (3rd Year) | Date: | 27/10/2026 |

Code of Conduct I G.Harshitha (192372384) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary action. Signature of the Student:

Instructions - Topic given before the session and students speak on stage. - Duration: 10 minutes (In class)

| Faculty In-charge | Course Coordinator | Program Director |
| --- | --- | --- |
| Signature: _______________ | Signature: _______________ | Signature: _______________ |
| Date: ___________________ | Date: ___________________ | Date: ___________________ |

Individual Task: Challenges in Mobile Computing and Cellular Networks

1. Introduction:

Mobile computing allows users to access information, applications and communication services anywhere and at any time while moving. It depends on cellular networks, which provide the wireless connection between the mobile device and the core network.

Even though mobile and cellular technologies have grown rapidly from 2G to 5G, they still face many technical and practical challenges because of the wireless medium, user mobility and limited resources.

2. Basic Overview:

A mobile computing environment can be represented as:

Mobile Device → Wireless Link → Base Station → Core Network → Internet / Other Networks

Challenges can occur at every stage of this chain: at the device, over the wireless link, at the base station, and in the core network.

3. Challenges in Mobile Computing:

These challenges are mainly related to the mobile device and the wireless environment.

| Challenge | Description |
| --- | --- |
| Limited battery power | Mobile devices run on batteries, and radio transmission, GPS, display and processing drain them quickly. Energy-efficient protocols and hardware are needed. |
| Limited resources | Processing power, memory, storage and screen size are small compared to desktops, so applications must be lightweight. |
| Mobility and handoff | A moving user must switch between cells or networks without losing the call or data session. |
| Variable bandwidth and connectivity | Wireless links have lower and less stable bandwidth than wired links, with frequent disconnections and high latency. |
| Security and privacy | The open wireless medium is prone to eavesdropping, spoofing, device theft and location tracking. |
| Heterogeneity | Different devices, operating systems and network technologies (2G/3G/4G/5G, Wi-Fi) must work together smoothly. |
| Location management | The network must continuously track where a user is so that calls and data can be delivered, which costs signalling overhead. |
| Data synchronization | Data must stay consistent when the device works offline and reconnects later. |

4. Challenges in Cellular Networks:

These challenges are related to the design, operation and security of the cellular infrastructure.

| Challenge | Description |
| --- | --- |
| Limited spectrum | Radio frequencies are a scarce, shared resource, so operators must serve more users with the same spectrum. |
| Interference | Co-channel and adjacent-channel interference between neighbouring cells reduces signal quality and capacity. |
| Signal fading and multipath | Buildings, terrain and user movement weaken and distort the signal, causing errors and poor quality. |
| Handover failures and call drops | Poor planning or weak signal at cell edges can cause calls and sessions to drop during handover. |
| Coverage gaps | Indoor areas, basements, hills and remote or rural regions often have weak or no coverage. |
| Capacity and congestion | Dense crowds, events and growing data demand overload base stations and the core network. |
| Network security threats | Threats include IMSI catching, fake base stations, SIM-swap, denial-of-service and signalling attacks. |
| High cost and energy use | Deploying base stations, backhaul and 5G small cells is expensive, and network equipment consumes a lot of power. |
| 5G-specific challenges | High-frequency mmWave signals have short range and are easily blocked, requiring dense small cells; network slicing and edge computing add management complexity. |

5. Possible Solutions:

- Power management: low-power hardware, sleep modes and energy-efficient protocols to extend battery life.

- Better spectrum use: frequency reuse, cell sectoring, carrier aggregation and Massive MIMO to increase capacity.

- Interference control: careful cell planning, power control and beamforming.

- Smooth mobility: improved handover algorithms and efficient location and mobility management.

- Stronger security: mutual authentication, encryption and secure 5G authentication (AUSF/UDM).

- Wider coverage: small cells, femtocells, Wi-Fi offloading and network densification.

- Edge computing and caching: processing data closer to the user to reduce latency and backhaul load.

6. Conclusion:

To conclude, mobile computing and cellular networks face challenges such as limited battery and resources, scarce spectrum, interference, mobility and handover issues, coverage gaps, and security threats. These can be reduced through efficient protocols, better network planning, advanced technologies such as Massive MIMO, small cells and edge computing, and strong security mechanisms. Addressing these challenges is essential for reliable, fast and secure mobile communication in 4G and 5G systems and beyond.

7. Geotag Photo: